Advanced Course

Mock

1/	Which is NOT licence for Amateur use

a/	5650 – 5670

b/	5765 –5820

c/	5820 – 5830

d/	5830 – 5850

2/	I am transmitting on 1.851 MHz maximum pep power shall be set to

a/ 32 dbw

b/ 15 dbw

c/  26 dBw

d/ 16dbw

3/	When must your antenna height NOT exceed 15 metres.

A/ When the local planning office decided

B/ When the station is within 1 mile of an aerodrome

C/ When the station is within 1 kilometre of an aerodrome

D/ When the deeds of the house specify

4/	You may not use your station for disaster communications,

A/ When approached by a member of the public.

B/ When approached by a member of the Health authority.

C/ When approached by a member of the police.

D/ When approached by a member of the fire service.

.

5/	What is the maximum time a greetings message can be transmitted from a club station to Canada?

A/	1min

B/	2min

C/	3min

D/	5 min

6/	On the 21st June you have a QSO with a VK station on 14 MHz at 8 AM UK local time. What time do you record in your logbook?

A/ 0700

B/ 0800

C/0900

D/ 2000

7/	If an authorised officer demands orally that you close down when should you comply?

A/ After 28 days

B/ After receiving the instruction in writing

C/ Immediately

D/ When you renew your licence.

8/	You have made an AMTOR QSO on 2 metres FM, which designator do you write in you log book.

A F1D

B/ F2B

C/ F1B

D/ F2D

9/	Which country can you NOT operate under the CEPT licence agreement?

A /	ZL

B/	VE

C/	W

D/	VK

10/	When operating /MM you should close down

A/ When contacted by a coast station

B/ During silence periods

C/ By order of the vessels Master.

D/ When requested by a passenger.

11/	Three resistor are connected in parallel of values 12 kohms, 6 kohms and 18 kohms, the total resistance is

A/ 32.7 kohms

B/	3.27 kohms

C/	7.2 kohms

D/	9kohms

12/	A 400pF and a 1600pF capacitor are connected in series the total capacitance is;

A]	160pF

B]	2000pF

C]	1200pF

D]	320pF

13/	If the spacing distance between the turns of an inductor is doubled does the inductance;

A/	Increase

B/ 	Decrease

C/ 	Double in value

D/	Has no effect

14/	The RMS value of a sine wave is;

A]	0.707 of peak voltage

B]	0.637 of peak voltage

C]	1.414 of peak voltage

D]	0.5 of peak voltage

- What is the phase relationship between the voltage and the current in a capacitor?

A]	The voltage lags the Current.

B]	The current lags the voltage

C]	Both in phase

D]	Voltage and current 180 degrees out of phase.

16.0 	An inductor of 25.5 H is connected in series with a capacitor of 20 pF, what is the approximate resonant frequency.

A]	14 MHz

B]	7 MHz

C]	14 kHz

D]	700 kHz

- The input impedance of a transformer is 50 ohms and the output impedance is 200 ohms. What is the turns ratio input to output of the transformer?

A]	1 : 2

B]	1 : 8

C 	4 : 1

D	1 : 4

18.0

Amplitude

Frequency

The drawing shows a;

A]	Band Stop filter.

B]	Low pass filter.

C]	High pass filter

D]	Band pass filter.

- To reduce the unwanted radiation from an oscillator into other stages of a receiver you would fit;

A]	Ferrite beads

B]	Decoupling Capacitors

C]	A metal screen

D]	An inductor.

20.0		In a semiconductor the boundary between the N and P type material is known as;

A]	The depletion layer

B]	Point barrier

C]	Screen layer

D]	Breakdown region.

21.0		VOLTS

TIME

The above waveform is the output from a;

A]	A high pass filter

B]	Low pass filter

C]	Half wave rectifier

D]	Full wave rectifier.

22.0

The above circuit represents a;

A]	An emitter follower

B]	A common base amplifier

C]	A mixer amplifier

D]	A common emitter amplifier.

23.0	To produce an output at 145 MHz a transmitter mixes to oscillator outputs of 110 MHz and 35 MHz. As well as the wanted output there is an unwanted output of:

A]	95 MHz

B]	75 MHz

C]	85 MHz

D]	70 MHz

24.0

The above drawing is a frequency synthesis, with some stages marked. What is box A?

A/	Phase comparator

B/	Crystal oscillator

C/ 	Low pass filter

D/	Divider

- On VHF to receive a FM signal the bandwidth of the receiver should be;

A/	300Hz

B/	10 – 16 kHz

C/	2.5 –3.0 kHz

D/	75kHz

- Which type of modulation requires a linear amplifier?

A/	SSB

B/	FM

C/	Phase

D/	CW

27.0

The circuit shown is;

A]	A low pass filter

B]	A key click filter.

C]	A CW oscillator

D]	A timing circuit.

28.0 	Parasitic oscillation may be caused by self resonance of;

A]	Non inductive Resistor.

B]	A diode.

C]	A transistor.

D]	An RF inductor.

29.0  	Which is a consequence of over driving a linear amplifier?

A]	The signal is difficult copy.

B]	The signal is attenuated.

C]	The signal occupies more bandwidth than necessary.

D]	The amplifier will fail.

- The Dynamic range of a receiver is;

A/	The signal to noise ratio

B/	The difference between the minimum signal and the onset of over loading

C/	The difference in output between minimum and maximum audio.

D/	The gain of the IF stage.

31.0

The partial completed block diagram is of superhet, what label should be in block A?

A/	Detector

B/	IF Amplifier

C/	RF Amplifier

D/	AGC Amplifier

32.0		Another name for second channel interference is;

A]	Blocking

B]	Image frequency

C]	Co Channel interference

D]	Spurious response.

- A double superhet receiver uses a high first IF because,

A/	To optimise adjacent channel interference..

B/	To reduce problems with the image frequency.

C/	Easier to constructed.

D/	Easier to receive FM transmissions.

34.0 	In a superhet receiver automatic gain control is provide to;

A] 	Enable operators to receive music.

B]	To copy CW transmissions

C]	To minimise the effects of fading.

D]	To provide a stable voltage to the local oscillator.

- To receive 430MHz band transmissions on a 2 metre receiver you require;

A/	A Down converter

B/	An Up converter.

C/	A Transverter

D/	 A wide band amplifier.

36.0

The balance line is connected to the following connections;

A]	2 & 3

B]	1 & 2

C]	2 & 4

D]	3 & 4

37.0		The overall length of a practical half wave dipole for 21.2 MHz when mounted at the correct height  is;

A]	14.15 metres

B]	7.075 metres

C]	6.721 metres

D]	7.500 metres

- In a trap dipole the trap is a;

A]	Capacitor

B]	Inductor

C]	A series tuned circuit of capacitance and inductance.

D]	A parallel tuned circuit of capacitance and inductance.

- You measure on your feeder system a forward power of 100 watts and a reflected power of 5 watts. What is the return loss.

A]	+13 dB

B]	-26 dB

C]	-13 dB

D]	+26 dB

- 40.0

The above circuit is of an Antenna Matching Unit, the circuit is known as;

A] 	A T match

B]	A series match

C]	A Pi match

D]	A L match

- 41.0 	The electromagnetic wave consists of;

A]	Radio waves

B]	E and H fields

C]	Polarised fields.

D]	E and M waves.

42.0	The sky wave is the normal propagation mode for contacts on;

A]	Daytime contacts on top band 160 metres.

B]	Night time contacts on 28 metres during sunspot minimum.

C]	Daytime contacts on 15 metres.

D]	Long distance contact on 2 metres.

43.0	Which amateur band is the best for daytime DX?

Critical				Maximum usable

Frequency			Frequency

a		b			c		d

A]	c

B]	a

C]	b

D]	d

44.0	Your own transmission on 18.134 MHz causes a problem on your TV receiver. You have already fitted ferrite rings to the main’s cable and a braid breaker to the antenna down lead. The most likely route into the TV receiver is therefore:

A]	IF pick up

B]	Base band pick up.

C]	Via DVD player

D]	Passive Intermodulation.

- You are causing TVI to a neighbours television on 2 metres, he is willing for you to examine his system. What is the most likely cause?

A]	Poor antenna mounting.

B]	His down lead.

C]	His mast head amplifier.

D]	The Television

- Your television picture consists of two identical images one fainter than the other. This is caused by;

A]	The TV signal arriving from two different directions.

B]	Cross modulation.

C]	Overloading of the front end.

D]	Blocking.

- A braid breaker filter is normally installed;

A]	As close to the TV antenna as possible.

B]	As close to the “RF input” of the system as possible.

C]	As close to the 13 amp mains as possible.

D]	Where it is hidden away.

48.0		RF ferrite beads a some times slipped over legs of transistors to cure;

A]	To improve stability

B]	To provide a DC path.

C]	To prevent Parasitic oscillations.

D]	To minimise harmonics.

- You are transmitting an ERP of 30 watts and an antenna gain of 3dBd what field strength will you have at the end of your garden 5 metres away?

A]	5.4 V/m

B]	10.8 V/m

C]	271 V/m

D]	54 V/m

- The worst antenna for TVI  is;

A]	A Doublet

B]	Monoband Yagi.

C]	A long wire.

D]	A Dipole.

- When should carry out tests on your station to prove you are not generating interference.

A]	From Time to time.

B]	Only when there is a complaint.

C]	When requested by an officer of OFCOM.

- D]	When instructed by the RSGB.

- Packet radio requires additional equipment called;

A]	Store and forward modem.

B]	Terminal Node Controller.

C]	A Mailbox.

D]	PSK Modem.

- Normal repeater shift on 2 metre’s is;

A]	110 Hz

B]	700 kHz

C]	600 kHz

D]	1MHz

- Special event stations shall be;

A]	Only operated by club members.

B]	Set up only by radio clubs.

C]	Restricted to local events.

D]	Open to viewing by members of the public.

55.0	On which band should we only transmit a CW signal?

A]	18.068 – 18.168

B]	1.850 -  2.000

C]	3.500 – 3.800.

D]	10.100 - 10.150

- Why should all exposed metal work be earthen?

A]	For Safety purpose..

B]	To reduce hand capacitance

C]	For screening purposes

D]	To protect equipment.

57.0	Which one of these is a mobile operators safety hazard.

A]	The equipment  placed on a seat to operate.

B]	The correct fuse is fitted.

C]	The power cables are routed correctly

D]	The operator uses a hand microphone.

58.0	A person should not be exposed to a power flux density is excess of;

A]	29 v/m

B]	28 v/m

C]	10 mw/cm

- D]	100 mw/cm

- An analogue meter can be extend to measure high currents by means of a;

A]	A series capacitor.

B]	A Shunt resistor.

C]	A parallel diode.

D]	A series Resistor.

- To confirm that a wavemeter is accurate you check against;

A]	Another wave meter

B]	you’re own frequency counter.

C]	A Standard radio Frequency transmission.

D]	Your oscilloscope.

61.0	What is the function of the time base control on an oscilloscope?

A]	Adjust the trace vertically.

B]	Attenuate the input signals.

C]	Determines the time the spot moves from left to right on the screen.

D]	Blanks the spot when it returns to the start position.

62.0	Measurement taken from an in - line Standing Wave Meter indicates a forward voltage of 6 volts and a reverse voltage of 4 volts. The VSWR on the line is;

A]	1.5 : 1

B]	1.6 : 1

C]	1.7 : 1

D]	1.8 : 1