Chelmsford Amateur Radio Society

EMC Intermediate Revision Questions

1/	To prevent RF entering the mains supply, we should fit a;

A/	An attenuator in the mains lead

B/	A high pass filter in the antenna feeder.

C/	A ferrite ring as a filter in the transceivers mains lead.

D/	A filter in all the audio leads.

2/	What constitutes a good RF earth?

A/	A wire in a tin of soil.

B/	a wire connected to the main’s earth.

C/	A wire touching the ground.

D/	A solid copper connection to one or more earth stakes.

3/	How would you check your station for harmonics and spurii?

A/	Ask another station whilst in QSO to check your station.

B/	Employ a local general coverage receiver to identified them.

C/	Employ a wavemeter.

D/	Employ a dip meter.

4/	Why are Loft antennas a potential source of EMC problems?

A/	Because they are enclosed.

B/	Because they are close to mains wiring and TV downleads.

C/	Because they are smaller than outdoor antennas.

D/	The feeder from the transmitter is not connected to earth.

5/	Why must we have an understanding of the subject of EMC?

A/	Because the operating of a transmitter in a domestic environment is not a normal situation.

B/	Because international regulation decrees it is part of the Examination.

C/	It is part of the OFCOM licence conditions.

D/	It is part of the RSGB licence conditions.

6/	What would be the most likely cause of interference from new consumer electronic equipment?

A/	the owner has purchased cheap equipment.

B/	It has been installed employing poor installation practise.

C/	Wrong mains voltage settings.

D/	Your receiver is too sensitive

7/	What is the best way to minimise EMC problems?

A/	Purchase the best equipment money can buy.

B/	Install the largest antenna possible.

C/	Correctly install your equipment and maintain good radio housekeeping.

D/	Operate as far away from the house as possible.

8/	On your TV receiver when you transmit you observe flickering picture and clicks on the sound. 
The most likely cause is;

A/	A FM transmission.

B/	A heating thermostat.

C/	A SSB transmission.

D/	A CW transmission.

9/	On your TV receiver screen you observe 2-10 seconds of intense spots and lines  
The most likely cause is;

A/	An electric motor.

B/	A CW transmission.

C/	A heating thermostat.

D/	Vehicle ignition.

10/	You have reported interference on a hi-fi system. Changing frequency has no affect. 
What is the most likely cause?

A/	Direct pick up on the speaker leads.

B/	Harmonics from the transmitter.

C/	Pick up on the hi-fi receiver intermediate frequency.

D/	Pick up on the hi-fi mains lead.

11/	Television interference occurs when you are operating on 2m and to a lesser extent when transmitting on 10 to 30 MHz. You are in a marginal television reception area. 
The most likely cause is;

A/	Direct pick up into the television.

B/	A Masthead amplifier.

C/	The antenna down lead.

D/	Mains pick up.

12/	What type of filter would you normally fit in a Television antenna co-axial downlead?

A/	Lowpass

B/	Bandpass

C/	Bandstop.

D/	Highpass.

13/	The connection of a dummy load to a transmitter, when investigating interference will remove from the investigation;

A/	All Harmonics.

B/	The RF field associated with the antenna.

C/	All mains borne interference.

D/	the mast head amplifier pick up.

14/	A station log is useful when dealing with an interference problem because;

A/	It helps the RSGB interference committee

B/	It will confirm the power used.

C/	It is an asset for your neighbour.

D/	Confirms’ the times of your transmission and therefore the possible times of television interference.

15/	The first point of contact for help and advice on interference problems is;

A/	OFCOM.

B/	The local EMC committee representative.

C/	Your local radio dealer.

D/	Your neighbour.