# Intermediate Mock Exam

# By Steve Hartley G0FUW

1 Which of the following callsigns would be a UK Intermediate Licence holder?

- M3QRP

- MM1QRZ

- 2U0QTH

- GI4QRX

2 An Intermediate Licence holder may operate using the callsign of:

- another Intermediate Licence holder without supervision

- another Intermediate Licence holder under the supervision of the other Licence holder

- a Full Licence holder under the supervision of the Full licence holder

- any amateur radio station under the supervision of the Licence holder

3 When operating from a hotel during a week’s holiday the temporary location of the Station must be given:

- every 15 minutes to an accuracy of at least 5km

- every 15 minutes to an accuracy of at least 10km

- every 30 minutes to an accuracy of at least 5km

- every 30 minutes to an accuracy of at least 10km

4 Which of the following is a Regional Secondary Locator for the United Kingdom?

- S

- D

- X

- G

5 Which of the following types of unattended operation is NOT permitted by the Intermediate Licence?:

- Beacon transmissions

- Leaving a data transmitter running whilst out at work

- The operation of a hidden transmitter intended for a direction finding contest

- Operation by another licensed radio amateur whilst you are out at work

6 How often must an Intermediate Licence holder test his/her station to ensure it is not causing ‘undue’ interference?

- from time to time

- daily

- weekly

- monthly

7 An amateur Licence must be renewed before:

- midday on the anniversary of the date of issue

- midday on the day before the anniversary of the date of issue

- the end of day on the anniversary of the date of issue

- the end of the day before the anniversary of the date of issue

8 What is the maximum permitted RF output power for an Intermediate Licence holder operating on 431.950MHz:

a.   1W erp

b.   32W

c.   40W

d.   50W

9 On which of the following Intermediate Licence frequency allocations must users accept interference from Industrial, Scientific and Medical users:

- 14.000 – 14.350MHz

- 24.890 – 24.990MHz

- 24000 - 24150MHz

- 248000 – 250000MHz

10 If a transmitter is rated at 10W input power and requires a 20V supply, what current will it draw:

- 0.5mA

- 500mA

- 2A

- 200A

11 If three 10kΩ resistors are wired in series their combined value will be:

- 1kΩ

- 3.3kΩ

- 10kΩ

- 30kΩ

12 A capacitor is best defined as:

- a number of turns of wire

- two metal discs separated by a thin layer of plastic

- a diode that can be used to set the frequency of a tuned circuit

- a semi-conductor

13 To change the resonant frequency of a tuned circuit you would need to change the:

- frequency of the oscillator

- value of the supply current

- value of the supply voltage

- combined values of the inductor and the capacitor

14 In a 24V mains power supply unit the transformer is used to:

- smooth the dc pulses

- change the alternating current to dc pulses

- reduce the 230V mains to 24V ac

- reduce the 230V mains to 24V dc

15 In a 24V mains power supply unit the rectifier diode is used to:

- smooth the dc pulses

- change the ac to pulses of dc

- reduce the 230V mains to 24V ac

- reduce the 230V mains to 24V dc

16 If a small base current flows in a transistor the collector current will be:

- equal

- larger

- smaller

- opposite

17 To measure current in a series circuit, the multi-meter test probes should be connected:

- in parallel with the relevant circuit

- in parallel with the component under test with the battery disconnected

- in series with the relevant circuit

- in series with the component under test with the battery disconnected

18 In a modern transmitter the balanced modulator is normally used to produce:

- single sideband

- two sidebands

- frequency modulation

- two tones for data transmissions

19 Mixing audio and radio signals together produces:

- sidebands

- harmonics

- oscillations

- spurious emissions

20 Harmonics from a transmitter are normally described as:

- reflections from aurora

- spurious emissions from transmitter

- interference from man-made sources

- multiples of the fundamental frequency

21 Which of the following responses would be produced by a high pass filter?:

22 The output from the demodulator in a superheterodyne receiver will be at the:

- audio frequency

- intermediate frequency

- wanted radio frequency

- local oscillator frequency

23 The wanted signals in a superheterodyne receiver are selected by tuned circuits in the:

- af amplifier

- if amplifier

- rf amplifier

- rf and if amplifiers

24 The purpose of a frequency discriminator is to:

- produce frequency modulation

- demodulate frequency modulation

- measure the frequency of an oscillator

- select the wanted frequency in an FM receiver

25 Coaxial feeder can be buried in damp ground because:

- the outer insulation acts as an rf screen

- the damp earth helps to ground the antenna

- radio frequency currents only exists between the feeder conductors

- the equal and opposite rf fields cancel each other out

26 If you find that you have a high SWR reading, you can reduced it by:

- shortening the feeder length

- tuning the antenna to match the frequency in use

- tuning the transmitter’s power amplifier to match the frequency in use

- adjusting the ATU to match the transmitter’s output impedance to the feeder’s characteristic impedance

27 If an Intermediate Licence holder feeds 16W of rf power into an antenna with a stated gain of 12dB the ERP will be:

- 1W

- 28W

- 192W

- 256W

28 Which of the following is the lowest in height:

- the troposphere

- the D layer of the ionosphere

- the E layer of the ionosphere

- the F layer of the ionosphere

29 Long distance VHF signals are normally propagated via:

- ground wave

- E layer reflection

- F layer reflection

- Moon bounce

30 Snow, ice or heavy rain would reduce propagation mostly on:

- LF

- HF

- VHF

- UHF

31 The output from commercial amateur radio transmitters normally cause interference because:

- they are poorly designed

- the do not have adequate filters

- EMC regulations do not apply to amateur equipment

- the rf fields generated are greater than the limits specified in EMC regulations

32 To check for over-modulation you need to:

- replace the transmitting antenna with a dummy load

- tune a sensitive receiver to multiples of the transmitter frequency

- tune a de-sensitised receiver to either side of the transmitter frequency

- replace the mains power supply unit with a battery

33 ‘Ghosting’ of a domestic television picture is likely to be a result of:

- amplitude modulated interference

- frequency modulated interference

- two pictures from the one transmitter being received at the same time

- an electrical motor (e.g. a hair drier) running near by

34 Voice signals from an amateur transmitter are being picked up on a CD player that has external speakers. You should fit:

- a high pass filter in the transmitter feeder

- a low pass filter in the transmitter feeder

- a ferrite ring on the mains cable of the CD player

- ferrite rings on all the leads of the CD player, including the external speaker leads

35 When investigating a complaint of breakthrough from your Station which of the following documents would be MOST useful?

- your logbook

- your TV Licence

- the Ofcom booklet BR68/I

- your current Validation Document

36 The code ‘QRZ?’ means:

- Who is calling me?

- What is your location?

- Do you have my details?

- Are you suffering from static noise?

37 Which of the following types of transmission is LEAST likely to be effective under rapidly changing propagation conditions?

- CW

- SSB

- FM

- SSTV

38 To enter an amateur radio contest you will normally need to:

- operate from an unusual location

- exchange specified information correctly

- produce QSL cards confirming all contacts

- be a member of the relevant national society

39 When using an amateur satellite to contact another country you must be able to receive:

- the up-link frequency

- the down-link frequency

- both the up-link and down-link frequencies

- the satellite’s beacon

40 When soldering, good eye protection is needed because:

- the tip of the soldering iron is very hot

- the solder fumes are corrosive to eyes

- getting solder fumes in your eyes can cause asthma

- molten solder could be flicked into the eyes causing blindness

41 When drilling through metal sheeting you should always:

a.   wear a tool belt

b.   hold tools by their blades

c.   hold the sheeting in a vice or clamp

d.   ensure that there is good ventilation

42 If you are fixing a new antenna to an existing mast you must:

- only use a metal ladder

- ensure that you are alone on the site

- set the ladder at the correct angle (4 up to 1 out)

- ensure that the ladder is placed well away from the antenna

43 A fuse is used to:

- produce inductance in a mains lead

- remove the mains supply in the event of too much current flowing

- remove the mains current if there is a very small current to earth

- protect against PME

44 ‘Tinning’ can be described as:

- scraping the bare wire to make it shine

- enclosing a tuned circuit with a tin shield

- ringing in the ears caused by listening to the radio at high volume

- coating a surface with solder before making the joint

45 A resistor with the coloured bands brown, black, black, silver would have a value close to:

- 0.1Ω

- 1Ω

- 10Ω

- 100Ω

NOW GO BACK AND CHECK YOUR WORK!

Intermediate Mock Exam Answers:

1	c

2	c

3	c

4	b

5	d

6	a

7	d

8	c

9	c

10	b

11	d

12	b

13	d

14	c

15	b

16	b

17	c

18	b

19	a

20	d

21	c

22	a

23	d

24	b

25	c

26	b

27	d

28	a

29	b

30	d

31	d

32	c

33	c

34	d

35	a

36	a

37	d

38	b

39	c

40	d

41	c

42	c

43	b

44	d

45	c