Chelmsford Amateur Radio Society  - Intermediate Revision Questions

1/	In a receiver why do we have AGC? Is it;

A/	To keep the RF amplifier stable

B/	To maintain frequency stability

C/	To maintain a near level audio output under all signals conditions

D/	To prevent the audio stage being over driven

2/	The AGC is normally applied to the;

A/	Front end mixer circuit.

B/	The IF stages

C/	The Demodulator

D/	Audio Stages

3/	A discriminator is used to recover the audio in a;

A/	SSB transceiver

B/	AM receiver

C/	FM receiver

D/	CW transmitter

4/	We wish to receive a signal on 7.050 MHz, the intermediate frequency in the receiver is 1.400 MHz. What frequency would the receiver local oscillator be tuned to?

A/	8.450 MHz

B/	14.100 MHz

C/	9.800 MHz

D/	2.800 MHz

5/	What is the most efficient mode of transmission;

A/	Amplitude Modulation

B/	Single Sideband

C/	Frequency Modulation

D/	Double sideband suppressed carrier

6/	An Amplitude Modulated signal contains;

A/	1 sideband and full carrier

B/	1 sideband and suppressed carrier

C/	2 sidebands and suppressed carrier

D/	2 sidebands and full carrier

7/	You are in QSO with a friend. Suddenly you have splatter interference from an adjacent QSO. The most likely cause of the splatter is;

A/	The adjacent station is running too much power

B/	The adjacent station has increased his microphone gain

C/	It is the 2nd Harmonic from the adjacent station

D/	Your receiver is faulty

8/	The purpose of a filter is to;

A/	To restrict a range of frequencies in and out of a circuit.

B/	To match between the stages.

C/	To select the required output frequency.

D/	To reduce interference.

9/	The main advantage of a crystal oscillator is;

A/	Easy to construct

B/	It has very good frequency stability

C/	Requires only a low supply voltage

D/	Is not affected by temperature changes

10/

Amplitude

Frequency

(A)

Amplitude

Frequency

(B)

Amplitude

Frequency

(C)

Amplitude

Frequency

(D)

Which diagram above represents the response of a Bandpass filter?

A/	(A)

B/	(B)

C/	(C)

D/	(D)

11/	Tuned circuits designed into receiver RF and IF amplifiers to;

A/	Provide matching between stages

B/	To select the wanted signals

C/	To increase power transfer

D/	To provide the correct voltages between stages

12/	We have a local oscillator on 9.5 MHz and the receiver front end is tuned to 7.1MHz.

The output of the mixer will contain output frequencies of;

A/	16.6 & 2.4 MHz

B/	23.7 & 4.7 MHz

C/	33.2 & 4.8 MHz

D/	14.2 & 19.0 MHz

13/

RF

Amplifier

DeModulator

BFO

Audio

Amplifier

Audio

The block diagram represents a;

A/	Superhet Receiver

B/	CW Receiver

C/	TRF or straight receiver

D/	A FM receiver

14/	To receive a CW Morse transmission your receiver will require;

A/	A beat frequency oscillator

B/	Modification

C/	An AGC Circuit

D/	A RF gain control

15/

Carrier   300                            3000

The diagram above represents;

A/	A double sideband transmission

B/	Upper sideband suppressed carrier signal

C/	Lower sideband suppressed carrier signal

D/	Amplitude modulated signal

RF

Oscillator

AF

Amplifier

Buffer

Amplifier

Power

Amplifier

Low Pass

Filter

Microphone

16/

The above block diagram represents a;

A/	SSB Transmitter

B/	FM Receiver

C/	AM Transmitter

D/	FM Transmitter